# OpenMANIPULATOR X Kinematics and Control Report

Stable project snapshot with calibrated FK IK motor limits and trajectory workflow

This report records the working calibration and mathematical model used by the current OpenMANIPULATOR X project. The snapshot uses kinematics version gripper-tip-v4-rotating-base-offset. The calibration is specific to the remounted robot and is not a universal ROBOTIS encoder-zero reference.

# Coordinate Frame and TCP

All positions are in millimetres. The base frame uses forward X, lateral Y, and upward Z. Joint 1 rotates the planar arm around the base vertical axis. The experiment uses the gripper tip or contact point as its TCP. The official ROBOTIS gripper-frame origin is 126.0 mm beyond joint 4, while the experimental TCP adds a measured 39.7 mm extension.

The working TCP distance from joint 4 is 165.7 mm.

The mathematical Z=0 plane is a base-model plane. It is not automatically the physical table surface.

The 12.0 mm joint 1 base offset rotates with q1 in both FK and IK.

# Geometry Measurements

| Symbol | Value | Meaning |
| --- | --- | --- |
| Z_BASE | 17.0 mm | Base vertical offset |
| L1_Z | 59.5 mm | Joint 2 vertical offset |
| Z0 | 76.5 mm | Z_BASE + L1_Z |
| BASE_X | 12.0 mm | Rotating joint 1 radial offset |
| L2_X and L2_Z | 24.0 mm and 128.0 mm | Joint 2 to joint 3 local offsets |
| L2 | 130.230565 mm | sqrt(24^2 + 128^2) |
| ALPHA2_0 | 79.380345 deg | atan2(128, 24) |
| L3_X | 124.0 mm | Joint 3 to joint 4 length |
| Official gripper frame | 126.0 mm | Joint 4 to ROBOTIS gripper frame |
| Tip extension | 39.7 mm | Gripper frame to experimental contact point |
| L4_X | 165.7 mm | Joint 4 to working TCP |

# Motor Encoder Conversion and Calibrated Angles

The DYNAMIXEL encoder has 4096 counts per revolution. Motor angles use the range 0 to 360 degrees. Joint differences use a wrapped range from -180 to less than +180 degrees.

mᵢ = ((rawᵢ / 4096) × 360) mod 360,    qᵢ = dᵢ wrap(mᵢ − hᵢ)

Encoder conversion and calibrated joint conversion

The inverse conversion adds the calibrated REST reference to the desired joint angle, wraps the result into 0 to 360 degrees, and converts the motor angle to a raw count with round(motor_deg / 360 x 4096).

| Motor | REST raw | REST motor angle | Direction |
| --- | --- | --- | --- |
| ID11 | 1917 | 168.486328 deg | +1 |
| ID12 | 2046 | 179.824219 deg | +1 |
| ID13 | 4049 | 355.869141 deg | +1 |
| ID14 | 0 | 0.000000 deg | +1 |

The straight raised REST pose defines q1 = q2 = q3 = q4 = 0. Wrapped subtraction correctly handles values close to the 0 or 360 degree encoder boundary.

# Named Poses

| Pose | Raw counts ID11 ID12 ID13 ID14 | Calibrated joints | Working TCP XYZ |
| --- | --- | --- | --- |
| REST straight raised | 1917, 2046, 4049, 0 | 0, 0, 0, 0 deg | 325.700, 0.000, 204.500 mm |
| WORK tool down | 1917, 2046, 4049, 943 | 0, 0, 0, 82.880859 deg | 180.536, 0.000, 40.077 mm |

At REST the official ROBOTIS gripper-frame origin is 286.000, 0.000, 204.500 mm. The working tip is farther forward because it includes the measured tip extension.

# Forward Kinematics

Convert calibrated q1 through q4 from degrees to radians as t1 through t4. The planar link directions and the radial and Cartesian equations are:

φ₂ = α₂₀ − t₂,    φ₃ = −(t₂ + t₃),    φ₄ = −(t₂ + t₃ + t₄)

Planar link directions

r = BASE_X + L₂ cos(φ₂) + L₃X cos(φ₃) + L₄ cos(φ₄)

Radial distance in the joint 1 plane

x = r cos(t₁),    y = r sin(t₁),    z = Z₀ + L₂ sin(φ₂) + L₃X sin(φ₃) + L₄ sin(φ₄)

Working TCP Cartesian forward kinematics

The working-tip FK uses L4 = 165.7 mm. The official ROBOTIS gripper-frame FK uses L4 = 126.0 mm. Joint 1 changes X and Y through the radial rotation. Joints 2, 3, and 4 all contribute to X and Z.

# Inverse Kinematics

The IK solver starts from the requested TCP XYZ and uses the actual current joint pose as the reference. It calculates joint 1 and the planar target, then tests multiple wrist angle candidates and both elbow branches.

t₁ = atan2(y, x),    rtarget = √(x² + y²) − BASE_X,    zplane = z − Z₀

Base rotation and planar target

A = L₃X + L₄ cos(t₄),    B = −L₄ sin(t₄),    Reff = √(A² + B²)

Effective two-link construction for a selected wrist angle

|L₂ − Reff| ≤ D ≤ L₂ + Reff,    D = √(rtarget² + zplane²)

Geometric reachability condition

α = arccos((L₂² + D² − Reff²) / (2 L₂ D)),    φ₂ = γ ± α,    t₂ = α₂₀ − φ₂

Elbow branch calculation

The solver reconstructs the remaining joint 3 angle from the elbow-to-target vector. Each candidate is converted to motor angles, checked against the Cartesian, joint, and motor limits, and reconstructed through FK. A candidate is accepted only when the FK position error is no greater than 2.0 mm.

The selected solution minimizes weighted squared wrapped motion from the reference pose:

C = 1.0 Δq₁² + 1.5 Δq₂² + 1.2 Δq₃² + 0.8 Δq₄²

Nearest valid joint configuration cost

This avoids choosing a fixed elbow configuration when another valid solution requires less movement from the current robot posture.

# Limits and Motion Checks

| Control | Minimum | Maximum or rule |
| --- | --- | --- |
| Cartesian X | 0 mm | 350 mm |
| Cartesian Y | -170 mm | +170 mm |
| Cartesian Z | No Cartesian box limit | Joint and IK checks still apply |
| q1 ID11 | -90 deg | +100 deg |
| q2 ID12 | -15 deg | +85 deg |
| q3 ID13 | -60 deg | +90 deg |
| q4 ID14 | -45 deg | +100 deg |
| ID11 through ID14 motor angle | 0 deg | 360 deg normalized |

IK position tolerance: 2.0 mm.

Python movement timeout: 30 seconds.

Post-move Python motor readback tolerance: 2.0 degrees.

Firmware final target tolerance: 8 encoder counts, approximately 0.7 degrees.

Firmware movement profile: cosine S curve, 20 ms step interval, 120 to 4500 ms duration.

# OpenCR and Serial Configuration

| Setting | Value |
| --- | --- |
| Controller board | ROBOTIS OpenCR |
| Validated PC port | COM7 |
| PC serial baud rate | 115200 |
| DYNAMIXEL bus | Serial3 |
| Direction pin | 84 |
| DYNAMIXEL bus baud rate | 1000000 |
| Protocol | 2.0 |
| Arm IDs | 11, 12, 13, 14 |
| Gripper ID | 15, excluded from arm TCP FK |
| Teaching sample interval | 50 ms or 20 Hz |
| Maximum teaching buffer | 30 seconds or 600 points |

The OpenCR firmware handles DYNAMIXEL input and output, torque, one-shot reads, interpolation, and teaching samples. Python owns calibrated FK, IK, workbook export, GUI behavior, and replay validation.

# Single Point Movement and Trajectory Replay

For a single XYZ command, enter X, Y, and Z, press PREVIEW NEAREST IK, review the target joints and delta joints, then press MOVE TO XYZ. The GUI requires confirmation before sending targets and reads the motors back after the motion.

Teaching and replay use separate paths:

motor angles → FK → XYZ      XYZ → IK → motor targets

Teaching stores Cartesian positions. Replay regenerates motor targets through IK.

The full workbook stores time, motor angles, calibrated joints, and XYZ for analysis. Replay reads only trajectory_xyz.xlsx and does not replay recorded joint columns directly. The playback solver uses the previous accepted IK posture as the next reference, preserving a near continuous joint path.

# Verification and Operating Notes

Offline checks cover REST FK, WORK FK, raw-degree conversion, motor-joint conversion, FK to IK to FK round trips, rotated base geometry, trajectory validation, and post-move target verification.

Close Arduino Serial Monitor before starting the Python GUI because both cannot use COM7 at the same time.

Clear the workspace and support the arm whenever the GUI requests it. Use STOP immediately if motion is unexpected.

If an Excel trajectory workbook is open, Windows may lock it. The program saves a matched timestamped workbook pair instead of overwriting the open files.

# Stable Snapshot Contents

code/python_app contains the Python GUI, calibrated kinematics, serial client, trajectory logic, and tests.

code/opencr_firmware contains the OpenCR Arduino sketch.

code/README.md contains operating instructions.

code/docs contains project planning notes.

documentation contains the Markdown and Word versions of this report.

No source code or kinematic equation was modified while creating this stable report and snapshot.